Appendix # 3.4

to the Annual Report of IDGC of Centre, PJSC

for 2015

Additional information on innovation development and R&D

The technical results delivered within the R&D implementation in the reporting period, developed equipment, technology, materials with visualization of the most significant results:

R&D «Development of anti-icing coatings and methods of their application to bare wires of Conductors based on ferromagnetic materials with Curie temperature close to 0°C». The project deliverables:

- Regional climatic zone maps for wind, ice and wind load at ice storm for 11 regions of IDGC of Centre (approved by Order of IDGC of Centre dated 20.01.2016 №12 TSA).

The purpose of the development of the maps is to increase the accuracy of the design calculations and loads on wires and poles of Conductors with the wind and the ice and as a consequence – the reliability of the line itself, which is especially important in regions with significant icing and wind loads. The maps are based on an analysis of long-term climate observations of weather stations located on the territory where the Company operates, the maps were reviewed by regional divisions of Hydrometeorological Centre of Russia, their application is planned in the design of new construction and reconstruction of Conductors from 2016.

- Experimental ferromagnetic coating composition coated on a bare conductor AS-120/19, as well as samples of aluminum rods having a cross section of 118 mm2.

Experimental ferromagnetic coating composition coated on a bare conductor AS-120/19

Experimental ferromagnetic coating composition coated on samples of aluminum rods having a cross section of 118 mm2

- Programs and Guidelines of preliminary environmental tests of the ferromagnetic coating.

- Scientific and technical report "Calculation of coating thickness range, ensuring the necessary level of heat sufficient to prevent icing".

- Scientific and technical report "Calculation of range to increase power losses and electric energy of wires coated with ferromagnetic coating in the working temperature, depending on the current load and the thickness of the coating".

- Scientific and technical Report "The results of calculation of the total annual average specific losses of Conductors coated with anti-icing-coated wires for the selected regions of IDGC of Centre".

- Technical and economic analysis of application of the ferromagnetic surface coating technology of Conductor wires used to prevent icing.

- The report on research patentability of the decisions taken on the composition and methods of forming the ferromagnetic coating on bare wires of used Conductors.

The list of received copyright protection (patents, certificates) on R&D results:

| R&D name, protected with documents of the exclusive right | Details of documents of the exclusive right | Details of the agreement, under which developed | Status of obtaining exclusive rights documents (copyright protection / application submitted) |
| --- | --- | --- | --- |
| Certificate of state registration of the computer program "Organization of the production activity of a company of the electric power industry" ("Modular industry solution: Production asset management system") | № 2015617601 dated 15.07.2015 | 7700/00153/12 dated 15.08.2012 | Copyright protection |
| Certificate of state registration of the computer program "Software for the analysis of performance of energy-efficient outdoor lighting systems" | № 2015614715 dated 11.03.2015 | 7700/000169/12 dated 20.09.2012 | Copyright protection |

Facilities at which the innovative solutions were implemented or transferred into trial operation in the reporting period:

| Item # | Branch of IDGC of Centre | Facility name | Key specifications |
| --- | --- | --- | --- |
| 1 | Belgorodenergo | Facilities of retrofitting and reconstruction, individual housing construction and grid connection in grids 0,4-10 kV | Power transformers 10 kV with reduced no-load and short-circuit losses: Oil-filled waterproofed transformer-10/0,4 40 kVА – 4 pcs. (Р no load=150 W, Psc=880 W); Oil-filled waterproofed transformer-10/0,4 63 kVА – 35 pcs. (Р no load=184 W, Psc=1460 W); Oil-filled waterproofed transformer-10/0,4 100 kVА – 28 pcs. (Р no load=280 W, Psc=2210 W); Oil-filled waterproofed transformer-10/0,4 160 kVА – 32 pcs. (Р no load=164 W, Psc=1822 W); Oil-filled waterproofed transformer-10/0,4 250 kVА – 12 pcs. (Р no load=425 W, Psc=3250 W); Oil-filled waterproofed transformer-10/0,4 400 kVА – 6 pcs. (Р no load=610 W, Psc=4600 W); Oil-filled waterproofed transformer-10/0,4 630 kVА – 3 pcs. (Р no load=800 W, Psc=6750 W) |
| 2 | Bryanskenergo | SS 35/6 kV Tembr | Vacuum recloser SMART 35 kV with auxiliary transformer – 5 pcs. Vn, kV - 35 Current rating, А - 1250 Current interrupting rating, kA - 20 Mechanical durability, CO - 20000 Сommutation life: - at Vn, cycles «CO» - 20000 - at Vn breaking, cycles «CO» - 25 Opening time, ms - 45 Total break time, ms - 55 Closing time, ms - 60 Resistance to lighting impulse - 190 kV Reclosing cycle О - 0,3 s – CO – 20 s – CO – 20 s - CO... Degree of protection provided by enclosures, GOST 14254-96 - IР55 Climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | Feeder №305 SS 110/6 kV «Soviet» | Recloser installation 10 kV РВА/TEL-10-12,5/630 U1 - 1 pcs. - Vn, kV - 10; - maximum voltage, kV - 12; - current rating, А – 630 - current interrupting rating, kA - 12,5 - short time electrodynamic current, kA - 32 - short-time thermal current, three seconds, kA - 12,5; - mechanical durability, cycles «CO» - 30000 - commutation life at current rating, cycles «CO» - 30000 - commutation life at current interrupting rating, cycles «CO» - 100 - climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | SS 110/35/6 kV «Dormash» | Power transformer monitoring system 63 MVA Power transformer monitoring system functions: - thermal monitoring of transformer condition; - control of currents and voltages HV, MV, LV; - oil level control; - control of moisture content and gas in the oil; - control of the current number of the tap of the tap changer; - bushing condition control; - control of degree of insulation aging and remaining life of the transformer; - cooling system control. Main unit (control of other modules, data acquisition, communication with the operator): - Vibration sensors - 4 pcs. - Pressure sensors - 6 pcs. - Current transducers - 3 pcs. - Voltage sensors - 3 pcs. - Humidity sensor - 1 pcs. - Temperature sensors - 7 pcs. Control unit of transformer bushings condition: - Set of control sensors of conduction currents and partial discharge. Control unit of insulation condition by partial discharge: - Partial discharge sensors – 15 pcs. Control unit of tap changer status: - Tap position sensor. - Current transducer «IFCT-5A». - Integrated sensor (temperature sensor, vibration sensor). In addition, a hydrogen and combustible gases analyzer "InteGaz" of the company INTERA is installed at the transformer Communication of the monitoring system with the computer is via RS-485 interface |
| 3 | Voronezhenergo | Construction of facilities of 10/0,4 kV within grid connection | Pole-mounted transformer SS 6(10)/0,4 kV S = 25 kVА (8 pcs.) S = 63 kVА (12 pcs.) Vn HV/LV, kV - 10/0,4; Maximum voltage of HV, kV - 12; Type – waterproofed; Cooling system - ONAN (oil); HV voltage regulation - ±2х2,5% (Rail NLTC); Short-time thermal current within 1s on HV, kA – 6,3; Type of low-voltage and high-voltage bushings – air (insulated), Sound pressure level, max, dBA – 40; Mounting of the power transformer on the pole of 10 kV Conductor - without additional supports (mounted version); HV surge protection - surge arrester as a part of the transformer (on the corrugated transformer tank cover) |
| 4 | Kostromaenergo | Construction of Conductor-0,4 kV to perform Grid connection contract № 41036336 with LLC CC "Volga-Stroy" | Steel multi-faceted pole UMz04-7-90: - quantity – 2 pcs. - height – 7 m; - wall thickness – 4 mm; - bending moment – 11,128 tfm - galvanized; - single circuit. Foundation for the pole: FT325х8х4000 - quantity – 2 pcs.; - pipe foundation; - pipe diameter – 325 mm; - pipe length – 4000 mm |
| 5 | Kurskenergo | Reconstruction of Conductor 110 kV «CHP-1 – Sadovaya» | Steel multi-faceted poles 110 kV: PM 110-4ph, (6 pcs.) - height 31,13 m; - bending moment 853,2 kNm PPM 110-2ph (30 pcs.) - height 30,418 m; - bending moment 853,2 kNm |
| 6 | Lipetskenergo | SS 35/10 kV «Black Sloboda» (Stage «D» EAD) | Switchgear HV – Switchgear and control gear D 40P, Switchgear LV – Switchgear and control gear D12P Relay protection and automation: ТОР 300 terminals Automatic system for commercial metering of power consumption: ARIS – EM Supervisory control system: ARIS – CS Equipment of Relay protection and automation and telemetry with the standard support IEC 61850 - cabinet of primary and backup protections Т1 and Т2 (2 pcs.). (Breakers Control Automation 35 kV implemented in the terminal of the transformer backup protection) - cabinet with automatic regulation of the transformation ratio (with mimic diagram) – 1 pcs. Process bus according to IEC 61850-8-1 Station bus according to IEC 61850-9-2LE Collection of digital signals of outgoing connections on the controller ARIS C 304 (25 pcs., including the integration of tap changers of transformers) Collection of measurement signals of CTs and VTs (for integration of analog circuits measuring differential transformer protection) – man-machine device (4 pcs.) Terminals of protection of outgoing connections (16 pcs.) Remote control server ARIS-SCADA NTP «Metronome» Server 600 Telecommunications cabinet Reservation of signals LAN Ethernet network – PRP technology Main communication channels with fiber optics using line link equipment «Polycom-300U-IGTR» Redundant communication channel – a small earth station VSAT satellite communications with an integrated Ethernet interface 100 Base-T |
| 7 | Smolenskenergo | Facilities of retrofitting and reconstruction, individual housing construction and grid connection in grids 0,4-10 kV | Pole-mounted transformer SS 6(10)/0,4 kV S = 63 kVА (11 pcs.) Vn HV/LV, kV - 10/0,4; Maximum voltage on HV, kV - 12; Type – waterproofed; Cooling system - ONAN (oil); HV voltage regulation - ±2х2,5% (Rail NLTC); Short-time thermal current within 1s on HV, kA – 6,3; Type of low-voltage and high-voltage bushings – air (insulated), Sound pressure level, max, dBA – 40; Mounting of the power transformer on the pole of 10 kV Conductor - without additional supports (mounted version); HV surge protection - surge arrester as a part of the transformer (on the corrugated transformer tank cover) |
|  |  | Reconstruction of SS 35 kV «Uspenskoye» | Vacuum recloser SMART 35 kV with auxiliary transformer – 1 pcs. Vn, kV - 35 Current rating, А - 1250 Current interrupting rating, kA - 20 Mechanical durability, CO - 20000 Сommutation life: - at Vn, cycles «CO» - 20000 - at Vn breaking, cycles «CO» - 25 Opening time, ms - 45 Total break time, ms - 55 Closing time, ms - 60 Resistance to lighting impulse - 190 kV Reclosing cycle О - 0,3 s – CO – 20 s – CO – 20 s - CO... Degree of protection provided by enclosures, GOST 14254-96 - IР55 Climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | Conductor-10 kV №1001/1002 SS «Zavodskaya» | Recloser installation 10 kV РВА/TEL-10-12,5/630 U1 - 1 pcs. - Vn, kV - 10; - maximum voltage, kV - 12; - current rating, А – 630 - current interrupting rating, kA - 12,5 - short time electrodynamic current, kA - 32 - short-time thermal current, three seconds, kA - 12,5; - mechanical durability, cycles «CO» - 30000 - commutation life at current rating, cycles «CO» - 30000 - commutation life at current interrupting rating, cycles «CO» - 100 - climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | SS 110 /10 kV «Yartsevo-2» | Equipment of Relay protection and automation and information acquisition & transmission system with the support IEC 61850 (integration into a single station bus IEC 61850-8-1) composed of: Cabinet of main and back-up protection of power transformers and 10 kV bushing – 2 pcs. Cabinet of control and automation of circuit breaker110 kV, sectionalizers 10 kV and tap changers of transformers – 1 pcs. Cabinet of central alarm equipment of the information acquisition & transmission system – 1 pcs. Terminals of protection of outgoing connections – 19 pcs. Additionally: - Computer workstation of a relay protection engineer; - digital data transmission channel to the top level supervisor Functionality of the information acquisition & transmission system: - collection and primary processing of analog and digital signals with input-output modules; - collection of information from microprocessor devices for digital communication channels; - transfer of information to the upper levels of supervisory control; - display of information directly on the substation on the Computer workstation of the relay protection engineer; - warning and alarm system; - registration and display of events; - continuous automatic monitoring of the status and functioning of hardware and software of both the information acquisition & transmission system, and the lower-level devices (Microprocessor relay protection and automation, communications, measurement); - setting microprocessor relay protection and automation devices and collection of waveforms, saving recorded waveforms in the archives; - support of staff to analyze waveforms; - information capacity of up to 5,000 signals; - capability to transmit information to the upper levels of control - up to 16 lines; - integration of up to 200 low-level devices. Functionality of relay protection and automation: - authorization of users (access levels and passwords) - emergency oscilloscope can record up to 12 analog channels and 64 digital signals - a complete set of relay protection for lines and inputs (standard configuration) - automatic re-closing modes and measurement for selected class of CTs and VTs - possibility of use of arc protection systems for cubicles (up to 3 sensors) and the GOOSE exchange between arc protection systems of different cubicles, which reduces the breaking time up to 10-15 ms - monitoring of the switching device Reservation: PRP with one Ethernet LAN ring |
| 8 | Tambovenergo | 35 kV SS 35/10 «Gorelskoye» | Vacuum recloser SMART 35 kV with auxiliary transformer – 1 pcs. Vn, kV - 35 Current rating, А - 1250 Current interrupting rating, kA - 20 Mechanical durability, CO - 20000 Сommutation life: - at Vn, cycles «CO» - 20000 - at Vn breaking, cycles «CO» - 25 Opening time, ms - 45 Total break time, ms - 55 Closing time, ms - 60 Resistance to lighting impulse - 190 kV Reclosing cycle О - 0,3 s – CO – 20 s – CO – 20 s - CO... Degree of protection provided by enclosures, GOST 14254-96 - IР55 Climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | SS 35/10 kV «Ekaterininskaya» | Resistive neutral grounding 10 kV Low resistor RZ-10 kV – 2 pcs. Resistance rating - 50 Ohm Power rating - 667 kW |
|  |  | Closed transforming substation-10/0,4 kV №4 F № 5(7) CDS-1 | Power transformer of amorphous steel core – 1 pcs. Unom – 10 kV, Transformer type – Oil-filled waterproofed transformer-SESCH-630/10-17, Power rating - 630 kVА, Nominal voltage of windings – 10/0,4 kV, Scheme and group of windings connection - D/Yn-11, No-load losses – 264 W, Short-circuit losses - 6555 W |
| 9 | Tverenergo | Facilities of retrofitting and reconstruction, individual housing construction and grid connection in grids 0,4-10 kV | Recloser installation 10 kV РВА/TEL-10-12,5/630 U1 - 9 pcs. - Vn, kV - 10; - maximum voltage, kV - 12; - current rating, А – 630 - current interrupting rating, kA - 12,5 - short time electrodynamic current, kA - 32 - short-time thermal current, three seconds, kA - 12,5; - mechanical durability, cycles «CO» - 30000 - commutation life at current rating, cycles «CO» - 30000 - commutation life at current interrupting rating, cycles «CO» - 100 climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | Conductor 110 kV «Bezborodovo-Redkino» (EAD stage) | Use of steel multifaceted poles 110 kV РM110-4PH (11902.00.000 MC) – 11 pcs. Type of pole: multifaceted intermediate double circuit with flange connection with structurally separate foundation Height: 31,13 m Weight: 3648 kg |
|  |  | Conductor 110 kV «Lazurnaya-Back Field» to SS Lebedevo (EAD stage) | Use of steel multifaceted poles 110 kV РM110-4PH (11902.00.000 MC). Unom of the line – 110 kV. Number of installed equipment – 48 pcs. Type of pole: multifaceted intermediate double circuit with flange connection with structurally separate foundation Height:31,13 m Weight: 3648 kg Construction length of the reconstructed section – 12,552 km |
| 10 | Yarenergo | Facilities of retrofitting and reconstruction, individual housing construction and grid connection in grids 0,4-10 kV (Nekrasovsky Distribution Zone) | Recloser installation 10 kV РВА/TEL-10-12,5/630 U1 - 1 pcs. - Vn, kV - 10; - maximum voltage, kV - 12; - current rating, А – 630 - current interrupting rating, kA - 12,5 - short time electrodynamic current, kA - 32 - short-time thermal current, three seconds, kA - 12,5; - mechanical durability, cycles «CO» - 30000 - commutation life at current rating, cycles «CO» - 30000 - commutation life at current interrupting rating, cycles «CO» - 100 climatic category and placement category under GOST 15150 - temperate and cold climate1 |
|  |  | Conductor 35 kV «Velikaya», Conductor 10 kV № 2 «Volkovsky» | Set of short-circuit indicators Fault Recorder-33 MP (including: three short-circuit indicators Fault Recorder-3 and main unit BSPI-3P) Conductor 10kV №2 «Volkovsky» SS Levoberezhnaya (Rybinsky Distribution Zone) – 3 pcs., Conductor 35kV Velikaya – 2 pcs. Functions: Types of faults: - interphase short-circuit detection, single phase-to-ground fault protection (metal); - separation of type of faults; - indication of fault phase Operations control: - visual (target, visibility 300 m); - radio unit; Current sensor – 3 pcs.; Voltage sensor – 3 pcs.; Response time to inrush current – 0,04 s; Fault analysis time - 0,1÷30 s Limits of threshold current - 10÷1000 А Limits of threshold voltage - 1÷35 kV Limits of threshold current at metal single phase-to-ground fault - 4÷40 А Additionally: - change of settings; - event log; - software update Power: Lithium storage battery – 16 Ah Fault Recorder weight - 0,8 kg Main unit BSPI-3P weight - 3 kg |
|  |  | Yargorelectroset | Universal charging station for electric vehicles - 5 pcs. Connector type: sockets Mennekes, Schuko, Type2 Vn 230 … 400 V Degree of protection IP44 Current rating 16/32 A Electric vehicle «Mitsubishi Outlander PHEV» - 1 pcs. Driving distance up to 880 km Consumption of 1,9 l/100 km when driving in the combined mode Plug - JEVS G105, CHAdeMO |
|  |  | Facilities of retrofitting and reconstruction, individual housing construction and grid connection in grids 0,4-10 kV | Pole-mounted transformer SS 6(10)/0,4 kV S = 63 kVА (3 pcs.) Vn HV/LV, kV - 10/0,4; Maximum voltage on HV, kV - 12; Type – waterproofed; Cooling system - ONAN (oil); HV voltage regulation - ±2х2,5% (Rail NLTC); Short-time thermal current within 1s on HV, kA – 6,3; Type of low-voltage and high-voltage bushings – air (insulated), Sound pressure level, max, dBA – 40; Mounting of the power transformer on the pole of 10 kV Conductor - without additional supports (mounted version); HV surge protection - surge arrester as a part of the transformer (on the corrugated transformer tank cover) |

The results of commercialization of the rights to intellectual property obtained as a result of R&D:

| Development name, protected with exclusive rights documents | Details the exclusive rights documents | Details of the contract, under which developed | Details of the license agreement / other information on use of the exclusive rights documents | Received payments, mln RUB, without VAT |
| --- | --- | --- | --- | --- |
| Patent for utility model of the pole mounted transformer substation | № 99905 dated 31.08.2010, № 101278 dated 31.08.2010, № 133982 dated 30.04.2013, № 146463 dated 27.01.2014 | 7700/00051/12 dated 11.04.2012 | № 7700/00074/15 dated 16.06.2015, № 7700/00150/15 dated 21.09.2015, № 7700/00158/15 dated 07.10.2015 | Planned receipt of payment in 2016 |
| Patent for utility model of the Power line pole (steel, multifaceted) | № 138695 dated 27.01.2014 | 7700/00193/13 dated 04.04.2013 | № 7700/00049/15 dated 15.04.2015 № 7700/00068/15 dated 02.06.2015 | Planned receipt of payment in 2016 |

The list of issues, reviewed by the Scientific and Technical Council of PJSC «Rosseti» in 2015:

| Item # | Meeting | Reviewed issues |
| --- | --- | --- |
| Presidium of the Scientific and Technical Council of PJSC «Rosseti» |  |  |
| 1 | 13.03.2015 | 1. Report on work of sections of the Scientific and Technical Council of PJSC «Rosseti» for 2014 2. On the concept of establishing a R&D fund 3. Approval of the work plan of the Scientific and Technical Council of PJSC «Rosseti» for 2015 |
| Section «Technologies and equipment of power lines» |  |  |
| 2 | 09.04.2015 | 1. Advantages of use of plastically pressed steel-aluminum wire and ground wire in the reconstruction and new construction of Conductors 2. Application at electric grid facilities of self-supporting non-insulated wire of flame-retardant grade SIPn 3. Consideration of the technical specifications for the R&D implementation «Development of the standard design of the transition point of 110 kV on the pole for connection of Conductor and Cable» 4. Consideration of the work plan of the section «Technologies and equipment of power lines» for 2015 |
| 3 | 20.05.2015 | 1. Creation of high-temperature superconductor of AC Cable with its length of 200 m at the voltage 20 kV with the current of 1500 Ai, installation in trial operation at the selected facility 2. Lead-in wires, terminals and couplings for high-temperature superconductor power lines 3. Compact, flexible high-current high-temperature superconductor cables and conductors for power transmission over short distances |
| Section «Technologies and equipment of substations» |  |  |
| 4 | 18.03.2015 | 1. Development and manufacturing of a commercial prototype of a phase-shifting device for the selected facility of the UNPG 2. Development of common technical requirements and standard technical solutions to the typical system of monitoring and diagnostics 3. On the work plan of the section «Technologies and equipment of substations» |
| 5 | 29.04.2015 | 1. Control of insulation of power transformers in the field on the basis of the PD monitoring system 2. Development of regulations of operation of power transformers with expired normative lifetime 3. Development and manufacturing of a developmental prototype of a SF6 power transformer 220 kV 63 MVA 4. Development of a methodology for assessing the technical condition of phase-insulated cast conductors on the basis of measurement and isolation of partial discharge, the study of magnetic fields and development of recommendations for laying and maintenance multiampere cast conductors 5. Technical requirements of PJSC «Rosseti» for controlled single-phase oil reactors of RUOM series of 6 - 10 kV |
| 6 | 10.06.2015 | 1. Resistive superconducting fault current limiters 2. Development and manufacture of a developmental prototype of transformer-resonance current-limiting device with a semiconductor switch (TOU-TRT) for 110 kV power line 3. Modular prefabricated substations based on SF6 Switchgear and control gear – technical solutions and performance evaluation |
| 7 | 24.07.2015 | Consideration of technical requirements of PJSC «Rosseti» for controlled single-phase oil reactors of RUOM series of 6 - 10 kV |
| 8 | 12.08.2015 | Federal testing centre. Main technical solutions |
| 9 | 23.09.2015 | 1. Development of fire protection of transformers using water mist technology 2. A comprehensive approach to improve the reliability of power transformers at short-circuit 3. Consideration of the implementation of technologies of processing the results of electrical contact surfaces with the use of gallium alloy |
| Section «Mode control, automation and use of automatic control in electric grids» |  |  |
| 10 | 02.07.2015 | 1. Development of a controller for connections of a digital substation with the support IEC 61850 in the domestic software and hardware platform 2. Development of a bus switch of a digital substation with the support IEC 61850 in the domestic software and hardware platform |
| Section «Strategic and system-wide issues and problems of functioning and development of electricity grids» |  |  |
| 11 | 28.04.2015 | 1. On the concept of use of new methods of spatial analysis of network systems for planning and spatial development of comparative analysis of power grids 2. Consideration of research results «Development of monitoring methodology of power consumption of regional electric power facilities» 3. Consideration of R&D topics «Use of synchronized vector measurement for the UNEG management», «Development of methods and software for calculation of modes of power distribution and location of commercial electricity losses» |